Name: Joseph MUTANGANA

Student ID: 29061

Course Name: Computer Networks

Instructor Name: Joshua IRADUKUNDA

Assignment Title: Assignment#1

Date: Sep-2025

HANDS-ON LAB

Prepared by: Joseph MUTANGANA

NETWORK CONFIGURATION LAB

IN CISCO PACKET TRACER

Table of Content

1. Introduction	1

2. Network Topology Design	1

3. IP Addressing Scheme and DHCP configuration	2

4. Routing between both LANs MASORO and GISHUSH	4

5. Remote Access Configuration (SSH)	5

6. NAT (PAT & Static NAT) Configuration	5

7. STP/RSTP and Port Security	6

8. HTTP Web Server Setup	7

9. Mail Server Configuration	7

10. Verification and Troubleshooting	8

11. Achieved Outcomes	11

12. Faced Challenges	11

Summary	12

# 1. Introduction

The purpose of the activity was to design, configure, and verify a small-scale network for two LANs — Masoro and Gishushu — connected through an ISP router, implementing core networking concepts used in real-world environments. The exercise focused on several key networking skills, including:

- Subnetting and IP addressing: Assigning unique IP subnets to different departments (IT, HR, Finance, CableNet, and WIFI-NET) and configuring routers and switches accordingly.

- DHCP configuration: Automating IP assignment for clients while excluding critical addresses such as the DNS server.

- Routing: Enabling communication between subnets using static routing.

- etc

# 2. Network Topology Design

Routers: Used to make connect network and Acts as DHCP server

Switch: Connects multiple end devices

End Devices (Server, PCs/Laptops): Clients used in topology

# 3. IP Addressing Scheme and DHCP configuration

Step 1: Opened Router in Packet Tracer -> Opened CLI tab

Step 2: Enter Configuration global mode

Step 3: Excluded IP address

I only had one Static IP address on server, this command ensures that this IP address exclude from DHCP pool

Step 4: Create DHCP Pool

Explanation of commands

enable: this enable enter EXEC mode.

configure terminal:  Enter global mode.

ip dhcp exclude-address 10.10.10.10: This tells dhcp pool to exclude that ip address.

ip dhcp pool IT-NET: This name of subnet you want pool address too.

network: Shows range of network and specify subnet mask

default-router: shows the gateway clients will use

dns-server: Sets domain name server

Step 5: Configure router interface

On Clients laptops to get IP address Dynamically

Step 1: Open laptop device

Step 2: Navigate to Desktop tab

Step 3: Select IP configuration

Step 4: Select DHCP

Verification by using the command prompt

Step 1: Open laptop device

Step 2: Navigate to Desktop tab

Step 3: Select command prompt

Step 4: type ipconfig or ipconfig /all

# 4. Routing between both LANs MASORO and GISHUSH

Step 1: Gave IP address to all subnets

Step 2: Assigned IP address on ISP

- Interface that face to MASORO-LAN

- Interface that face to GISHUSHU-LAN

Step 3: Applied IP Route commands on GISHUSH-ROUTER

Step 4: Applied IP Route commands on MASORO-ROUTER

Step 3: Applied IP Route commands on ISP-ROUTER

ip route 10.10.10.0 255.255.255.0 200.200.200.2:  This tells Gishushu router to use 200.200.200.2 port faces to ICP to send traffic to network has this IP 10.10.10.0 in Masoro LAN, and vice versa.

# 5. Remote Access Configuration (SSH)

Explanation of used Commands

ip domain-name jmutangana.rw: defines the domain name for the router because SSH requires a domain to generate RSA encryption keys.

Username: Allows to set name of user who is using the device.

Secrete: Allows to set secrete for user when try log in.

Crypto key generate rsa: Generates RSA key used for SSH encryption.

ip ssh version 2: Enable SSH version 2

line vty 0 4: Enters configuration for virtual terminal lines 0 to 4

transport input ssh: Restrict remote to SSH only.

login local: tells the router/switch to use local username and password

# 6. NAT (PAT & Static NAT) Configuration

Step 1: Define inside and Outside Interface

ip nat outside: Marks the WAN interface as outiside

ip nat inside: Marks the LAN interfaces as inside for NAT

Step 2: Create an Access List to define inside Address

Access-lists 11 permit 10.10.10.0 0.0.0255: Defines which private IPs are allowed to be translated (Masoro’s LANs).

access-lists 11 deny 10.10.10.10 0.0.0255: Excludes the server IP from translation

Step 3: Apply NAT Overload (PAT)

ip nat inside source list 11 interface g9/0 overload: Tells the router to use the public IP of the outside interface (g9/0) for all internal users

overload: Enable PAT (Port Address Translation) many private IPs share one public IP, using different port numbers

7. STP/RSTP and Port Security

Explanation of used commands

Interface range FastEthernet0/2 – 6: This allows to apply configure same configuration on more than one port.

switchport mode access: forces port to work as access port.

switchport access vlan 13: Assign the port to vlan 13.

switchport port-security: Enables port security features on the port.

switchport port-security maximum 1: Limits each port to learn only one MAC addresss

switchport port-security violation shutdown: If violation occurs, the port goes into error.

switchport port-security mac-address sticky: Allows switch to store first connected MAC address in the running configuration

# 8. HTTP Web Server Setup

Step 1: Open Server

Step 2: Go to service tab

Step 3: Select HTTP on left side

Step 4: Enable on button on HTTP

# 9. Mail Server Configuration

Step 1: Open Server

Step 2: Go to service tab

Step 3: Select EMAIL on left side

Step 4: Enable (on) button to open SMTP

Step 5: Enter Domain Name

Step 6: Click set button

Step 7: Create user by Entering username and password

Step 8: Click on Plus Button on right side

# 10. Verification and Troubleshooting

1.Device information

2. Interface & IP Verification

3. Routing Verification

4.Nat Verification

5. DHCP Verification

6. Access Control List (ACL) Verification

7.Vlan & Trunk Verification

8. Port Security Verification

9.Connectivity Tests

10. Saving & Exiting

Explanation of used Commands

show-running-config:  Displays current configuration

show startup-config: Display saved configuration

show ip interface brief: Gives summary of all interface

show interfaces: Displays detailed info about every interface

show ip route: Shows the routing table, including directly connected.

show protocols: Lists all interfaces with their assigned IPs and protocol status

show ip nat translations: Displays the active NAT translation

show ip nat statistics: Displays how many translations exist.

show ip dhcp pool: Shows all configured DHCP pools with details

show ip dhcp binding: Shows current IPs leased to clients

show access-lists: List all ACLs configure on the devices

show vlan brief: shows all vlan configure on the switch

show interface trunk: verifies interface are operating as trunks

show interfaces switchport: shows switchport mode

show port-security: This summarize port-security status for all interfaces.

chow port-security interface fa0/1: Display detailed port-security info for specific port

ping: used to test network reachability

copy running-config startup-config: Saves current configuration startup-config.

write: Saving configuration

write memory: Saving configuration

end: exiting any configuration mode.

exit: Exist the CLI

# 11. Achieved Outcomes

Connectivity Masoro ↔ Gishushu achieved.

Web server reachable on all clients.

Mail server operational across campuses.

Secure management via SSH.

VLANs, STP, and port security correctly configured.

# 12. Faced Challenges

In the Gishushu LAN, I initially struggled with assigning IP addresses to WIFI-NET (10.10.14.0/24) and CABLE-NET (10.10.13.0/24) because both networks were connected to the same switch, which in turn had only one physical link to the router. At first, I tried assigning only one IP address to the router port, but this did not allow both networks to communicate properly.

Later, I discovered the concept of Router-on-a-Stick, which allows a single physical router port to handle multiple IP subnets by creating sub-interfaces, each assigned to a specific VLAN. I configured the switch port connecting to the router in trunk mode, enabling it to carry traffic for multiple VLANs simultaneously. I then applied NAT on these sub-interfaces to allow the devices in both VLANs to access external networks.

Using this method, a single router port was able to serve multiple IP flows corresponding to their respective VLANs, resolving the connectivity issue and reinforcing my understanding of inter-VLAN routing and VLAN tagging.

I corrected by following these steps

Step 1: Configure sub-Interface

Explanation of used Commands

Interface g8/0.13: creates a sub-interface .13 on physical interface g8/0

Interface g8/0.14: creates a sub-interface .14 on physical interface g8/0

Encapsulation dot13Q:  Assigns VLA 13 to this sub-interface using 802.1Q trunking.

Ip address 10.10.13.1 255.255.255.0: Assigns IP address for CABLE-NET (VLAN 13)

Ip nat inside: Marks sub-interface as inside for NAT

exit: Exit from sub-interface

Step 2: Switch port Trunking

Explanation of use commands

Interface g0/1: entering the port connected to router

switchport mode trunk: Configure the switch port connectivity to router as trunk.

switchport trunk allowed vlan 13,14: Restrict the trunk to only VLAN 13 and VLAN 14

# Summary

During the activity, the following tasks were successfully completed:

- IP Addressing & Subnetting:

  - Masoro LAN: IT-NET (10.10.10.0/24), HR-NET (10.10.11.0/24), FINANCE-NET (10.10.12.0/24)

  - Gishushu LAN: CABLE-NET (10.10.13.0/24), WIFI-NET (10.10.14.0/24)

  - All subnets configured with mask 255.255.255.0, DNS server 10.10.10.10.

- DHCP Configuration:

  - Created pools for each subnet.

  - Excluded critical IPs (like the DNS server).

  - Configured default gateways and DNS settings for automated client IP assignment.

- Routing: Static routing enabled on both Masoro and Gishushu routers to ensure inter-subnet communication and internet access via the ISP.

- VLANs & Trunking:

  - Configured VLANs on the S4 switch.

  - Access ports assigned to appropriate VLANs.

  - Trunk links set to carry multiple VLANs between switch and router.

- Port Security:

  - Applied to all access ports, limiting the number of MAC addresses per port.

  - Sticky MAC addresses enabled and violation action set to shutdown.

- SSH Configuration:

  - Local users created and encrypted passwords configured.

  - RSA keys generated, SSH version 2 enabled.

  - VTY lines configured for SSH login only.

- Used commands like show ip interface brief, show running-config, show vlan brief, show port-security, show ip nat translations, and ping to confirm network connectivity, security, and proper configuration.

END